`timescale 1ns/1ps

module seat_detect(input clkin,input [3:0] val,input reset,output reg [6:0]up_led,output reg [6:0]down_led);

parameter both_empty = 2'b00;

parameter up_full = 2'b10;

parameter down_full = 2'b01;

parameter full = 2'b11;

reg [1:0]cur_state;

reg [2:0] up,down;

initial begin

cur_state <= both_empty;

up<=3'b111;

down<=3'b111;

end

wire [3:0] pb_out;

debounce db_0(val[0],clkin,pb_out[0]);

debounce db_1(val[1],clkin,pb_out[1]);

debounce db_2(val[2],clkin,pb_out[2]);

debounce db_3(val[3],clkin,pb_out[3]);

clock_div clk_1(clkin,clk);

always @ (posedge clk)

begin

if(reset == 1)

begin

up<=3'b111;

down<=3'b111;

end

else if(pb_out[0] == 1)

begin

case(cur_state)

full:

begin

up<=3'b000;

down<=3'b000;

end

up_full:

begin

up<=3'b000;

if(down == 3'b1)

begin

down<=down-3'b1;

cur_state<=full;

end

else

begin

down<=down-3'b1;

end

end

down_full:

begin

down<=3'b000;

if(up == 3'b1)

begin

up<=up-3'b1;

cur_state<=full;

end

else

begin

up<=up-3'b1;

end

end

both_empty:

begin

if(down == 3'b1)

begin

cur_state<=down_full;

end

down<=down-3'b001;

end

endcase

end

else if(pb_out[1] == 1)

begin

case(cur_state)

full:

begin

down<=down+3'b1;

cur_state<=up_full;

end

down_full:

begin

down<=down+3'b1;

cur_state<=both_empty;

end

up_full:

if(down != 3'b111)

down<=down+3'b1;

both_empty:

if(down != 3'b111)

down<=down+3'b1;

endcase

end

else if(pb_out[2] == 1)

begin

case(cur_state)

full:

begin

up<=3'b000;

down<=3'b000;

end

up_full:

begin

if(down == 3'b1)

begin

down<=down-3'b1;

cur_state<=full;

end

else

begin

down<=down-3'b1;

end

end

down_full:

begin

if(up == 3'b1)

begin

up<=up-3'b1;

cur_state<=full;

end

else

begin

up<=up-3'b1;

end

end

both_empty:

begin

if(up == 3'b1)

begin

up<=up-3'b1;

cur_state<=up_full;

end

else

begin

up<=up-3'b1;

end

end

endcase

end

else if(pb_out[3] == 1)

begin

case(cur_state)

full:

begin

up<=up+3'b1;

cur_state<=down_full;

end

up_full:

begin

up<=up+3'b1;

cur_state<=both_empty;

end

down_full:

if(up != 3'b111)

up<=up+3'b1;

both_empty:

if(up != 3'b111)

up<=up+3'b1;

endcase

end

end

always @(posedge clk)

begin

case (up)

0 : up_led = 7'b1111110;

1 : up_led = 7'b0110000;

2 : up_led = 7'b1101101;

3 : up_led = 7'b1111001;

4 : up_led = 7'b0110011;

5 : up_led = 7'b1011011;

6 : up_led = 7'b1011111;

7 : up_led = 7'b1110000;

default : up_led = 7'b1111111;

endcase

case (down)

0 : down_led = 7'b1111110;

1 : down_led = 7'b0110000;

2 : down_led = 7'b1101101;

3 : down_led = 7'b1111001;

4 : down_led = 7'b0110011;

5 : down_led = 7'b1011011;

6 : down_led = 7'b1011111;

7 : down_led = 7'b1110000;

default : down_led = 7'b1111111;

endcase

end

endmodule

module debounce(input pb_1,clk,output pb_out);

wire slow_clk;

wire Q1,Q2,Q2_bar;

clock_div u1(clk,slow_clk);

my_dff d1(slow_clk, pb_1,Q1 );

my_dff d2(slow_clk, Q1,Q2 );

assign Q2_bar = ~Q2;

assign pb_out = Q1 & Q2_bar;

endmodule

// Slow clock for debouncing

module clock_div(input Clk_100M, output reg slow_clk);

reg [26:0]counter=0;

always @(posedge Clk_100M)

begin

counter <= (counter>=249999)?0:counter+1;

slow_clk <= (counter < 125000)?1'b0:1'b1;

end

endmodule

// D-flip-flop for debouncing module

module my_dff(input DFF_CLOCK, D, output reg Q);

always @ (posedge DFF_CLOCK) begin

Q <= D;

end

endmodule